# Dynamic range compression

Dynamics compression is very commonly used in musical production and game audio. It lowers the volume of the loudest parts of the signal while boosting the overall level. Used well, it can achieve a louder, punchier, and fuller sound. Dynamic range compressor is one of the more complicated audio nodes, especially when compared to the other classic audio effects (gain, delay, stereo panning, biquad filters…). But it is often overused, increasing distortion and reducing the intended dynamics in audio content.

Dynamic audio effects apply a time varying gain to the input signal. The applied gain is typically a nonlinear function of the level of the input signal (or a secondary signal). Dynamic effects are most often used in order to modify the amplitude envelope of a signal. They often compress or expand the dynamic range of a signal.

In this Chapter, we explain the theory of dynamic range compression and its application in the Web Audio API with the DynamicsCompressorNode. A large portion of the chapter deals with understanding the parameters and how their use affects the sound. However, the DynamicsCompressorNode is not fully specified in the Web Audio API. Hence the first source code example shows how to visualize the effect of varying the ballistics, that is, the compressor's attack and release parameters. The second code example shows a compressor in action. It displays a meter of the gain reduction while a user changes the parameters of a compressor that is applied to a source.

## Introduction

Dynamic range compression (or just compression) is concerned with mapping the perceived dynamic range of an audio signal to a smaller perceived range. Dynamic range compressors achieve this goal by reducing the high signal levels while leaving the quieter parts untreated. It is especially important in music and games where large numbers of individual sounds are played simultaneously, in order to control the overall signal level and help avoid clipping (distorting) the audio output to the speakers. Dynamic range compression should not be confused with data compression as used in audio codecs, which is a completely different concept.

A compressor is essentially a variable gain control, where the amount of gain used depends on the level of the input. Attenuation is applied (gain less than one) when the signal level is high, which in turn makes louder passages softer, reducing the dynamic range.

There is no single correct form of dynamic range compressor implementation, and there are a bewildering variety of design choices. This explains why every compressor in common usage behaves and sounds slightly different and why certain compressor models have become audio engineers' favorites for certain types of signal. Analysis of compressors is difficult because they represent nonlinear time-dependent systems with memory. The gain reduction is applied smoothly and not instantaneously as would be the case with a simple static nonlinearity. Furthermore the large number of design choices makes it nearly impossible to draw a generic compressor block diagram that would be valid for the majority of real world compressors. Some differ in topology, others introduce additional stages and some simply differ from the precise digital design since these deviations add character to the compressor. However we can describe the main parameters of a compressor unit and specify a set of standard stages and building blocks that are present in almost any compressor design.

Our goal here is to describe how a compressor is designed, and then discuss the specific implementation in the Web Audio API, considering both technical and perceptual aspects. We first provide an overview of the basic theory, paying special attention to the adjustable parameters used to operate a dynamic range compressor. Then the principles of its operation are described, including detailed discussion of digital implementation. Next, different methods and implementations are given for a complete design. Applications of compressors are then discussed, along with the artifacts that may result with their use.

## Theory

A compressor has a set of controls directly linked to compressor parameters through which one can set up the effect. The most commonly used compressor parameters may be defined as follows.

Threshold defines the level above which the compressor is active. Whenever the signal level overshoots this threshold, the  level will be reduced.

Ratio controls the input/output ratio for signals overshooting the threshold level. It determines the amount of compression applied. A ratio of 3:1 (‘three to one’) implies that the input level needs to increase by 3 decibels in order for the compressor to apply a 1 decibel reduction.

A compressor's input/output relationship is often described by a simple graph, as Figure 13.1. The horizontal axis corresponds to the input signal level, and the vertical axis is the output level, where level is measured in decibels. A line at 45 degrees through the origin corresponds to a gain of one, so that any input level is mapped to exactly the same output level. The ratio describes how the compressor changes the slope of that line above the threshold value. The distance between the highest and lowest output levels defines the dynamic range of the output.

Limiting is simply an extreme form of compression where the ratio is very high (often described as 20:1 or higher), and thus the input/output relationship become very flat. This places a hard limit on the signal level.

Attack and release times, also known as time constants, control the speed at which a compressor reacts to a change in signal level. Instantaneous compressor response is not usually sought because it introduces distortion on the signal.

The attack time defines the time it takes the compressor to decrease the gain to the level determined by the ratio once the signal overshoots the threshold. The release time defines the time it takes to bring the gain back up to the normal level once the signal has fallen below the threshold.

A Make-Up Gain control is usually provided at the compressor output. The compressor reduces the level (gain) of the signal, so that applying a make-up gain to the signal allows for matching the input and output loudness level.

The Knee Width controls whether the bend in the response curve has a sharp angle or has a rounded edge. The Knee is the threshold-determined point where the input-output ratio changes from unity to a set ratio. A sharp transition is called a Hard Knee and provides a more noticeable compression. A softer transition where, over a transition region on both sides of the threshold, the ratio gradually grows from 1:1 to a user defined value is called a Soft Knee. It makes the compression effect less perceptible. Depending on the signal one can use hard or soft knee, with the latter being preferred when we want less obvious (transparent) compression.

In order to smooth the transition between compression and no compression at the threshold point, we can soften the compressor's knee. The width W of the knee (in decibels) is equally distributed on both sides of the threshold. Figure 13.1 presents a compression gain curve with a soft knee.

Commercial compressors often have additional controls such as hold and side-chain filtering. However, here we focus only on the parameters exposed in the Web Audio API’s DynamicsCompressorNode.

Figure 13.1. Static compression characteristic with make-up gain and hard or soft knee. When the input signal level is above a threshold, a further increase in the input level will produce a smaller change in the output level.

### A simple dynamic range compressor

Here, we present a simple yet high quality dynamic range compressor, to illustrate how they can be designed.

First, we estimate signal level using a simple decibel representation of the instantaneous signal magnitude,

| ydB=20log10¦x¦ | (13.1) |
| --- | --- |

This is then used as input to the gain computer, xG=ydB, where the gain is calculated as,

| yG=xGxG+(1/R-1)(xG-T+W/2)2/(2W)T+(xG-T)/R 2(xG-T)≤-W2¦(xG-T)¦<W2(xG-T)≥W | (13.2) |
| --- | --- |

Where T is the threshold, W is the knee width, and R is the ratio. Smoothing with attack and release times is accomplished by applying attack and release time constants to the output of the gain stage. So

| xB=xG-yG yB[n]=αAyB[n-1]+(1-αA)xB[n]αRyB[n-1]+(1-αR)xB[n] xB[n]>yB[n-1]xB[n]≤yB[n-1] | (13.3) |
| --- | --- |

Finally, the gain is applied,

| cdB=-yB c=10cdB/20 yn=xn⋅cn | (13.4) |
| --- | --- |

This compressor was described in (Giannoulis, et al., 2012) and we will return to it in Chapter 19. However, the Web Audio API’s DynamicsCompressorNode, as we shall see, is quite a different implementation.

## DynamicsCompressorNode

The DynamicsCompressorNode is the Web Audio API’s default audio node for dynamic range compression. Unlike, most nodes, the DynamicsCompressorNode provides a lot of flexibility in terms of how it is implemented. That is, many details are left unspecified.

The DynamicsCompressorNode has the following parameters,

It has five adjustable parameters.

- threshold,  between -100 and 0 dB with default value -24. The decibel value above which the compression will start taking effect.

- ratio,  between 1 and 20 with default value 12 dB. The amount of dB change in input for a 1 dB change in output.

- knee,  between 0 and 40 dB, with default value 30. A decibel value representing the range above the threshold where the curve smoothly transitions to the "ratio" portion.

- attack,  default 0.003 with maximum value 1. The amount of time (in seconds) to reduce the gain by 10dB.

- release,  default 0.25 with maximum value 1. The amount of time (in seconds) to increase the gain by 10dB, with max value 1

In addition, there is one parameter which can be read but not set.

- Reduction: The current amount of gain reduction, in decibels, that the compressor is applying to the signal. If fed no signal the value will be 0 (no gain reduction). This is useful for metering purposes.

### Under the hood of the DynamicsCompressorNode

Dynamics compression can be implemented in a variety of ways. The DynamicsCompressorNode has the following characteristics:

- Fixed look-ahead (the compressor adds a fixed latency to the signal chain).

- Configurable attack speed, release speed, threshold, knee hardness and ratio.

- Side-chaining is not supported.

- The gain reduction is reported via the reduction property on the DynamicsCompressorNode.

This curve is continuous and piece-wise differentiable, and corresponds to a target output level, based on the input level.

The compression curve has three parts: no change to the input, f(x)=x; the knee, which is a monotonically increasing function, and a linear gain reduction, f(x)=x/ratio.

Internally, the DynamicsCompressorNode is described with a combination of other AudioNodes, as well as an algorithm to compute the gain reduction. Figure 13.2 is used internally, input and output respectively being the input and output AudioNode, and a new class, EnvelopeFollower, that instantiates a special object that behaves like an AudioNode, described below:

const delay = new DelayNode(context, {delayTime: 0.006});

const gain = new GainNode(context);

const compression = new EnvelopeFollower();

input.connect(delay).connect(gain).connect(output);

input.connect(compression).connect(gain.gain);

Figure 13.2. graph of internal AudioNodes used as part of the DynamicsCompressorNode processing algorithm. It implements pre-delay and application of gain reduction.

Due to the look-ahead delay, this node will output non-silent audio with zero input for some time.

The Envelope Follower is not fully specified in the Web Audio API specification. Rather, there is some general guidance and a few requirements while the rest is left open to interpretation. This approach is somewhat justified since unlike many other audio effects, there aren’t formal rules for compressor design. However, it is also problematic since, without knowing what’s going on under the hood, there’s no guarantee that the compressor will do what you think it should.

The guidelines include the fact that the EnvelopeFollower is based on applying the input-output characteristic curve to the input signal, in order to produce a signal level for each sample of input. The compression curve is defined such that it is identity up to the threshold, then the soft knee region is from threshold to threshold + knee. The shape of the curve over the knee width is unspecified. Above this region, it is again linear, with slope 1/ratio.

Attack and release are based on the time it takes to decrease or increase the gain by 10 dB. Since, in most designs this also depends on other compressor parameters, the meaning of both of these is almost completely open. Note that one can have a compressor that performs an adaptive release, that is, releasing faster the harder the compression, or have curves for attack and release that are not of the same shape.

The makeup gain is an internal variable, not a user-adjustable parameter. It is a fixed gain stage that only depends on ratio, knee and threshold parameters, and not on the input signal. The intent is to increase the output level of the compressor so it is comparable to the input level.

Makeup gain = (1/ gfr)0.6, where gfr is the full range gain, i.e  the result of applying the compression curve to 1 (0 dB).

Further aspects to note are that side-chaining is not supported, and it has a fixed look-ahead so it adds a latency to the signal chain.

### Example- attack and release times

<canvas id='canvas' width=600 height=300 style='border:1px solid'></canvas><br>

<button id='Start' onclick='audioContext.resume()'>Start</button><br>

<input type='range' min=0 max=1 value='0.1' step='any' id='Attack'>Attack<br>

<input type='range' min=0 max=1 value='0.1' step='any' id='Release'>Release<br>

<button id='Show'>Show</button>

<script>

let canvasContext= canvas.getContext('2d')

let audioContext= new AudioContext()

let source= audioContext.createConstantSource()

source.start()

let compressor= audioContext.createDynamicsCompressor()

source.connect(compressor).connect(audioContext.destination)

Attack.oninput = () => compressor.attack.value = Attack.value

Release.oninput = () => compressor.release.value = Release.value

</script>

<script src='AttackRelease.js'></script>

function linearTodB(x) { return 20*Math.log10(x) }

function dBtoLinear(x) { return Math.pow(10,x/20) }

Show.onclick = function() {

let now = audioContext.currentTime

source.offset.setValueAtTime(dBtoLinear(-20), now)

source.offset.setValueAtTime(1, now + 0.33)

source.offset.setValueAtTime(dBtoLinear(-20), now + 0.67)

canvasContext.clearRect(0, 0, canvas.width, canvas.height)

var x,xPrev,yIn,yInPrev,yOut,yOutPrev

var xScale = canvas.width, yScale = - canvas.height / 30

function plotLoop() {

xPrev=x, yOutPrev=yOut, yInPrev=yIn

x = xScale*(audioContext.currentTime - now)

yIn = yScale*linearTodB(source.offset.value)

yOut = yIn + yScale*compressor.reduction

canvasContext.beginPath()

canvasContext.strokeStyle = 'black'

canvasContext.moveTo(xPrev,yInPrev)

canvasContext.lineTo(x,yIn)

canvasContext.stroke()

canvasContext.beginPath()

canvasContext.strokeStyle = 'purple'

canvasContext.moveTo(xPrev,yOutPrev)

canvasContext.lineTo(x,yOut)

canvasContext.stroke()

if (audioContext.currentTime<now+1)  requestAnimationFrame(plotLoop)

}

plotLoop()

}

Code example 13.1. AttackRelease.html and AttackRelease.js, for visualizing the effect of changing Attack and Release parameters.

Code example 13.1 shows the effect of varying attack and release parameters. A DynamicsCompressorNode is used with the default parameters, which are quite good for most purposes.

The input to the compressor is a ConstantSourceNode whose offset parameter jumps to 1 at 0.25 seconds, then jumps back down to 0 at 0.75 seconds. A canvas is used to plot the input in black and the compressor's reduction parameter (the compressor's ) in purple.

Roughly, the compressor should be in the attack phase while the input is at a high value, and the release phase while the input is at a low value. The user can adjust the attack and release parameters using sliders, to change the duration of both phases.

### Example: compressor parameters

<button onclick='context.resume()'>Start</button>

<button active='false' id='compress'>Add compression</button><br>

<canvas id='canvas' width='10' height='100' ></canvas><br>

<input id='Threshold' type='range' min=-100 max=0 value=-20>Threshold<br>

<input id='Ratio' type='range' min=1 max=20 value=12>Ratio<br>

<input id='Knee' type='range' min=0 max=40 value=0>Knee<br>

<input id='Attack' type='range' min=0 max=1000 value=0>Attack<br>

<input id='Release' type='range' min=0 max=1000 value=5>Release

<script src='compressor.js'></script>

let context= new AudioContext()

const source1 = new OscillatorNode(context)

const source2 = new OscillatorNode(context,{frequency:0.25})

const gainNode = new GainNode(context)

const compressor = new DynamicsCompressorNode(context,

{threshold:-20,knee:0,ratio:12,attack:0.002,release:0.005})

source1.start()

source2.start()

source1.connect(gainNode)

source2.connect(gainNode.gain)

gainNode.connect(context.destination)

let canvasContext= canvas.getContext('2d')

function draw() {

requestAnimationFrame(draw)

canvasContext.clearRect(0,0,canvas.width,canvas.height)

canvasContext.fillRect(0,100*(1+compressor.reduction/25),5,100)

}

draw()

compress.onclick = function() {

if(compress.getAttribute('active') === 'false') {

compress.setAttribute('active', 'true')

compress.innerHTML = 'Remove compression'

gainNode.disconnect(context.destination)

gainNode.connect(compressor).connect(context.destination)

} else {

compress.setAttribute('active', 'false')

compress.innerHTML = 'Add compression'

gainNode.disconnect(compressor)

compressor.disconnect(context.destination)

gainNode.connect(context.destination)

}

}

Threshold.oninput = () => compressor.threshold.value = Threshold.value

Ratio.oninput = () => compressor.ratio.value = Ratio.value

Knee.oninput = () => compressor.knee.value = Knee.value

Attack.oninput = () => compressor.AttackRelease.value = Attack.value

Release.oninput = () => compressor.release.value = Release.value

Code example 13.2. Compressor.html and Compressor.js, showing a dynamic range compressor in action. The user can control all parameters and a meter depicts the amount of gain reduction at any time.

Code example 13.2 shows a compressor in action. It plots the amount of gain reduction currently applied by the compressor to the signal. The input is an oscillator modulated by a very low (0.25 Hz) frequency oscillator, having the effect of the amplitude reaching an extreme every 2 seconds. This is then connected to a DynamicsCompressorNode, with a button to disconnect this and connect the modulated oscillator straight to the destination. A simple level meter shows the amount of gain reduction, read from the compressor’s reduction property.

Here, we’ve exposed all the compressor parameters, so you can hear the effect of changing them, as well as visualize it with the meter. To observe more extreme compression, set the ratio high and the threshold low, with attack and release set to 0. You should hear a very buzzy distortion, which can be reduced in different ways by adjusting the parameters back to more typical values. On the other hand, setting attack and release to near their maximal values drastically reduces the effect of compression. High attack and release times mean that the compressor’s gain changes act too slowly to modify the dynamics and instead act like a gradual adjustment to a volume control..

## Misuse and overuse of dynamic range compression

### Artifacts

There are a large number of artifacts associated with compressors. These are primarily to do with parameter settings that result in unwanted modification of the signal. To name just a few:

- dropouts – these are ‘holes,’ or periods of unwanted near silence, in the output signal that result from a strong attenuation immediately after a occur short, intense sound;

- pumping – perceived variation of signal level, as a result of reducing gain as signal crosses threshold and turning up as signal dips below;

- breathing – similar to pumping, this is perceived variation of background noise level due to rapid  gain changes in conjunction with high background noise;

- modulation distortion- distortion that is caused by the gain control changing too rapidly;

- spectral ducking - broadband gain reductions in the processed signal that occur as a result of a narrow-band interfering signal; and

- SNR reduction - reduction in the signal-to-noise ratio caused by boosting low-level (noise signals) signals and/or attenuating high level sources.

Drop-outs, pumping and breathing are functions of the attack and release time changes, and to some extent, the threshold. Pumping and breathing, in particular, are quite well-known and sometimes used intentionally. Sometimes, especially with dance music, the producer may want an audible change every time the compressor ‘kicks in.’ A short attack or release time can be used in order to achieve a quick change in the gain and achieve modulation of the overall signal level. This gives a 'pumping' sound.

When the sound level drops below the threshold, the gain increases with a rate dependent on the release time. With appropriate choice of time constants, the signal level can remain reduced even though the input level is low. Thus, the noise can be made audible, giving the impression of breathing. A more sophisticated compressor may watch the input closely and adjust the gain when the input hits zero momentarily to reduce the 'breathing' effect. Since breathing involves raising the noise floor, it could be considered a subset of SNR reduction.

Generally, a compressor should leave the power spectrum unchanged. If our input is a pure sinusoid, with a sudden change in level, then the spectrum should be a single peak, with a tiny bit of harmonic distortion in just the window where the transition occurred.

### Loudness wars

Dynamic range compression is used at almost every stage of the audio production chain. It is applied to minimise artifacts in recording (like variation in loudness as a vocalist moves towards or away from a microphone), to reduce masking and to bring different tracks into a comparable loudness range. Compression is also applied in mastering to make the recording sound ‘loud.’ since a loud recording will be more noticeable than a quiet one, and the listener will hear more of the full frequency range. This has resulted in a trend to more and more compression being applied, a ‘loudness war.’

Broadcasting also has its loudness wars. Dynamic range compression is applied in broadcasting to prevent drastic level changes from song to song, and to ensure compliance with standards regarding maximum broadcast levels. But competition for listeners between radio stations has resulted in a trend to very large amounts of compression being applied.

So a lot of recordings have been compressed to the point where dynamics are compromised, transients are squashed, clipping occurs and there can be significant distortion throughout. The end result is that many people think, compared to what they could have been, a lot of modern recordings sound terrible. And broadcast compression only adds to the problem.

Who is to blame? There is a belief among many that ‘loud sells records.’ This may not be true, but believing it encourages people to participate in the loudness war. And each individual may think that what they are doing is appropriate. Collectively, the musician who wants a loud recording, the record producer who wants a wall of sound, the engineers dealing with artifacts, the mastering engineers who prepare content for broadcast and the broadcasters themselves are all acting as soldiers in the loudness war.

The tide is turning

The loudness war may have reached its peak shortly after the start of the new millenium. Audiologists became concerned that the prolonged loudness of new albums might cause hearing damage. Musicians began highlighting the sound quality issue, and the European Broadcasting Union addressed the broadcast loudness wars with EBU Recommendation R 128 and related documents that specify how loudness and loudness range can be measured in broadcast content, as well as recommending appropriate ranges for both.

Together, all these developments may go a long way to establishing a truce in the loudness war.