# The wonders of audio worklets

In the previous chapter, we saw how to create audio worklet nodes that have all the functionality of the default nodes in the Web Audio API. In fact, the audio worklet allows one to do almost any audio processing that one might wish to do in a development environment.

In this last chapter, we revisit some previous examples, and show how they can be achieved using audio worklets instead. This allows us to circumvent issues with some of the audio nodes. Code examples are given to show how pulse waves and square waves may be created without use of a Fourier series, the bit crusher can be designed without storing a wave shaping curve, a dynamic range compressor may be implemented with very fast response, and a stereo enhancer can preserve signal power. And finally, with careful use of circular buffers, the Karplus-Strong algorithm may be implemented without strong limitations on the pitch range for the sound that is produced.

## Example – pulse waves and square waves revisited

The square wave was used to introduce a lot of concepts. A square wave was constructed as a default type of the OscillatorNode or using the PeriodicWave to set Fourier coefficients for a square wave in Chapter 2. In Chapter 3, we processed the output of an AudioBufferSourceNode that contains one period of a waveform to create a square wave. In Chapter 4, we created square waves by setting the frequency parameters of many OscillatorNodes.

In Chapter 2 we also introduced the pulse wave, which is like a square wave but has an additional parameter, the duty cycle, allowing one to vary the proportion of each period that the wave spends near its high value. By setting the duty cycle to 0.5, a square wave is produced.

However, all of these approaches involve the Fourier series approximation to a square wave (though the approach in Chapter 3 did not need to do this). That is, the square wave is constructed by summing sine waves, such that the resulting approximation has no frequencies above Nyquist. This is preferred if one wants to avoid aliasing, but there are many applications where an actual square wave might be preferred. For instance, it may be used as a test signal, or to turn a sound on and off, or as the basis for an alarm or alert sound.

An additional limitation for all of the default oscillator types is that one cannot control their phase. This is not problematic in most situations. And there are ways around this, such as muting the source with a gain node until the current phase reaches the preferred value, but it still would be nice to easily set the initial phase of an oscillator.

These limitations are addressed in Code example 19.1, which shows how audio worklets can be used to generate a pulse wave oscillator. The initial phase is given as a processorOption and accessed with a number control, since for this example, it is only used when the node is initialized. However, it could have been made an audio parameter. We track the phase as the signal is generated, to determine when to switch between high and low states. If duty is set to 0.5, the resulting waveform is an ideal square wave. If duty is set close to 0, the resulting waveform is a pulse train with short periodic pulses in an otherwise audio stream.

<button onclick=context.resume()>Start</button><br>

<input type=range min=0 max=2000 value=440 step='any' id='Freq'>Freq<br>

<input type=range min=0 max=1 value=0.5 step='any' id='Duty'>Duty cycle<br>

<input type=number min=0 max=360 value=0 step='any' id='Phase'>Phase

<script>

let context= new AudioContext()

context.audioWorklet.addModule('PulseWorklet.js').then(() => {

let pulse = new AudioWorkletNode(context,'pulse-generator',

{processorOptions:{phase:Phase.value}}

)

pulse.connect(context.destination)

Freq.oninput = () => pulse.parameters.get('frequency').value=Freq.value

Duty.oninput = () => pulse.parameters.get('duty').value=Duty.value

Phase.oninput = () => pulse.parameters.get('phase').value=Phase.value

})

</script>

registerProcessor('pulse-generator',class extends AudioWorkletProcessor {

constructor(options) {

super()

if (typeof options.processorOptions !== 'undefined') this.phase = 0

else this.phase = options.processorOptions.phase / 360

}

static get parameterDescriptors() { return [

{name:'frequency',defaultValue: 440},

{name:'duty',defaultValue:0.5,max:1,min:0},

{name:'phase',defaultValue:0,max:360,min:0}

]}

process(inputs, outputs, params) {

for (let channel = 0; channel < outputs[0].length; ++channel) {

if (params.frequency.length === 1) {

for (let i = 0; i < outputs[0][channel].length; ++i) {

if (this.phase > params.duty[0]) outputs[0][channel][i]=-1

else outputs[0][channel][i]=1

this.phase += params.frequency[0] / sampleRate

this.phase = this.phase-Math.floor(this.phase)

}

} else {

for (let i = 0; i < outputs[0][channel].length; ++i) {

if (this.phase > params.duty[i]) outputs[0][channel][i]=-1

else outputs[0][channel][i]=1

this.phase += params.frequency[i] / sampleRate

this.phase = this.phase-Math.floor(this.phase)

}

}

}

return true

}

})

Code example 19.1. Pulse.html and PulseWorklet.js, which use an audio worklet to create a pulse wave.  By setting the duty cycle to 0.5, this also creates square waves. Note that here we do not attempt to avoid aliasing.

## Example – the bit crusher revisited

In Chapter 12 we saw how to create a bit crusher, that changes the apparent number of bits used to encode a signal. One awkward aspect of that implementation is that if one changes the bit depth, one needs to create a new curve for the wave shaper. These curves require a fair amount of storage and computation since, for instance, 16 bit encoding requires at least 216 data points in the curve. An alternative would be to store a set of curves, one for each possible bit depth, but that would also be cumbersome and wasteful.

We can avoid these issues by instead using an audio worklet. Rather than using different waveshaping curves to determine how each sample is processed, we simply have the processing in the audio worklet processor depend on a bit depth audio parameter. This is shown in Code example 19.2.

<button onclick=context.resume()>Start</button>

<script>

const context = new AudioContext()

context.audioWorklet.addModule('bitCrushWorklet.js').then(() => {

let osc = new OscillatorNode(context)

let bitcrusher = new AudioWorkletNode(context,'bitcrusher')

osc.connect(bitcrusher).connect(context.destination)

osc.start()

})

</script>

registerProcessor('bitcrusher', class extends AudioWorkletProcessor {

static get parameterDescriptors () {

return [{name: 'bitDepth',defaultValue: 12,minValue: 1,maxValue: 16}]

}

process (inputs, outputs, parameters) {

const input = inputs[0],output = outputs[0]

const bitDepth = parameters.bitDepth

const frequencyReduction = parameters.frequencyReduction

if (bitDepth.length > 1) {

// The bitDepth parameter array has 128 sample values.

for (let channel = 0; channel < output.length; ++channel) {

for (let i = 0; i < output[channel].length; ++i) {

let step = Math.pow(0.5, bitDepth[i])

output[channel][i] = step * Math.floor(input[channel][i] / step + 0.5)

}

}

} else {

// bitDepth is constant, so put computation of step outside the loop

const step = Math.pow(0.5, bitDepth[0])

for (let channel = 0; channel < output.length; ++channel) {

for (let i = 0; i < output[channel].length; ++i) {

output[channel][i] = step * Math.floor(input[channel][i] / step + 0.5)

}

}

}

return true

}

})

Code example 19.2. BitCrusher.html, using an audio worklet to quantize a signal.

## Example – dynamic range compression revisited

As mentioned in Chapter 13, the DynamicsCompressorNode is not clearly specified in the Web Audio API. Its also a fairly complicated one, with an inherent latency of at least one frame. Also in Chapter 13, we gave the equations for a simpler dynamic range compressor. This can be implemented in an audio worklet, thus allowing the developer to have an easily modifiable implementation with a fast response. This is shown in Code example 19.3.

<button onclick='context.resume()'>Start</button><br>

<input id='Threshold' type='range' min=-100 max=0 value=-30>Threshold<br>

<input id='Ratio' type='range' min=1 max=20 value=12>Ratio<br>

<input id='Knee' type='range' min=0 max=40 value=0>Knee<br>

<input id='Attack' type='range' min=0 max=1000 value=0>Attack<br>

<input id='Release' type='range' min=0 max=1000 value=5>Release

<script src='compressor.js'></script>

let context= new AudioContext()

const source1 = new OscillatorNode(context)

const source2 = new OscillatorNode(context,{frequency:0.25})

const gainNode = new GainNode(context)

source1.start()

source2.start()

source1.connect(gainNode)

source2.connect(gainNode.gain)

context.audioWorklet.addModule('compressorWorklet.js').then(() => {

let compressor = new AudioWorkletNode(context,'compressor')

gainNode.connect(compressor).connect(context.destination)

Threshold.oninput = () =>

compressor.parameters.get('threshold').value = Threshold.value

Ratio.oninput = () => compressor.parameters.get('ratio').value = Ratio.value

Knee.oninput = () => compressor.parameters.get('knee').value = Knee.value

Attack.oninput = () => compressor.parameters.get('attack').value = Attack.value

Release.oninput = () => compressor.parameters.get('release').value = Release.value

})

registerProcessor('compressor', class extends AudioWorkletProcessor {

static get parameterDescriptors() { return [

{name:'threshold',defaultValue:-30},

{name:'ratio',defaultValue:12},

{name:'knee',defaultValue:0},

{name:'attack',defaultValue:0},

{name:'release',defaultValue:5}

] }

constructor() {

super()

this.yPrev = 0

this.port.onmessage = () => this.port.postMessage(this.reduction)

}

process(inputs, outputs, parameters) {

let alphaAttack=Math.exp(-1/(parameters.attack[0]*sampleRate/1000))

let alphaRelease=Math.exp(-1/(parameters.release[0]*sampleRate/1000))

let x,x_dB,y_dB,x_G,y_G,x_B,y_B,c_dB,c

let T=parameters.threshold[0]

let R=parameters.ratio[0]

let W=parameters.knee[0]

for (let j=0;j<outputs[0][0].length;++j) {

// dB conversion

x_dB=inputs[0][0][j]

y_dB=Math.max(10*Math.log10(x_dB*x_dB),-120)

// gain computer

x_G=y_dB

if ((x_G-T)<= -W/2) y_G=x_G

else if ((x_G-T)>= W/2) y_G=T+(x_G-T)/R

else y_G=x_G+(1/R-1)*(x_G-T+W/2)*(x_G-T+W/2)/(2*W)

//ballistics

x_B=x_G - y_G

if (x_B>this.yPrev) y_B = x_B * (1-alphaAttack) + alphaAttack * this.yPrev

else                y_B = x_B * (1-alphaRelease) + alphaRelease * this.yPrev

this.yPrev=y_B

// gain stage

c_dB = -y_B

this.reduction = c_dB

c= Math.pow(10,c_dB/20)

outputs[0][0][j]=c*inputs[0][0][j]

}

return true

}

})

Code example 19.3. Compressor.html, using an audio worklet to apply dynamic range compression.

This implementation does not apply a make up gain, since this is not relevant for all uses of a compressor, it can be implemented with a gain node following the compressor, and a reasonable value is hard to calculate and should depend on the input.

Note that we included a reduction parameter, as with the DynamicsCompressorNode. This is not necessary, but useful for easy metering. Since it is not an audio parameter, we access it using the message ports of the AudioWorkletNode and AudioWorkletProcessor.

## Example – stereo enhancer revisited

The stereo enhancer, or stereo widener, was introduced in Chapter 16. However, a problem with that implementation was that signal power is not maintained. To do that, one needs to know the initial panning position of the stereo source for any given sample. One can use the AnalyserNode to get this, but it’s a bit of a hack, introduces latency, and then one still needs to apply a time varying gain to preserve the signal power. However, this is easily achieved if we use an audio worklet to create the stereo enhancer, as shown in Code example 19.4. Here, we use a variable norm to ensure that power is maintained as the stereo width is narrowed or widened.

<input type='range' min=-1 max=1 value=0 step='any' id='width'>

<script>

let context= new AudioContext()

context.audioWorklet.addModule('stereoWidenerWorklet.js').then(() => {

let Widener= new AudioWorkletNode(context,'stereo-widener')

let monoSource = context.createOscillator()

let source = new StereoPannerNode(context,{pan:0.5})

source.pan.value = 0.5

monoSource.connect(source)

source.connect(Widener)

monoSource.start()

Widener.connect(context.destination)

width.oninput = () => {

context.resume()

Widener.parameters.get('width').value= width.value

}

})

</script>

registerProcessor('stereo-widener', class extends AudioWorkletProcessor {

static get parameterDescriptors() { return [

{name:'width',defaultValue:0,minValue:-1,maxValue:1}

]}

constructor(options) {

super()

options.numberOfOutputs=2

}

process(inputs, outputs, parameters) {

const input = inputs[0],output = outputs[0]

for (let channel = 0; channel < input.length; ++channel) {

for (let i = 0; i < input[channel].length; ++i) {

let L,R,M,S,newM,newS,newL,newR,W,norm

L=input[0][i]

if (input.length>1) R=input[1][i]

else R=input[0][i]

M = (L+R)/Math.sqrt(2) // obtain mid-signal from left and right

S = (L-R)/Math.sqrt(2) // obtain side-signal from left and right

W= (parameters.width[0]+1)*Math.PI/4

newM= M*Math.cos(W)

newS= S*Math.sin(W)

newL= (newM+newS)/Math.sqrt(2)

newR= (newM-newS)/Math.sqrt(2)

norm= Math.sqrt((L*L + R*R) / (newL*newL+newR*newR))

output[0][i]= norm*newL

output[1][i]= norm*newR

}

}

return true

}

})

Code example 19.4. stereoWidenerNode.html and stereoWidenerWorklet.js, using an audio worklet to apply stereo width enhancement.

## Circular buffers and delay lines

As we have seen, audio worklet nodes takes a block of 128 samples as input, perform some processing, and output 128 samples. This process is then repeated for the next, nonoverlapping block of 128 samples.

A similar procedure is used for most audio applications, where processing is done frame by frame or block by block. In Digital Audio Workstations (DAWs), there is usually a default buffer size which gives the number of samples in a frame. This can usually be set by the user, and common values are 32, 64, 128, 256, 512, 1024 and 2048. Note also, that the time duration of this frame is also dependent on the sampling frequency. For instance, at 44,100 kHz, 128 samples is 128/44,100 = 2.90249… ms duration.

In many situations, if you want to access the signal at a previous time, you can use the delay node. But often, one wants to access precise input samples. In that case, one might want to work with a frame (or block) of samples of a different duration, or just access samples outside the current block. For instance, suppose one wants to store the last 12 samples. That’s easy to do if you are looking at sample 12 in the block; the last 12 samples are samples 0 to 11. But what if you are accessing the first sample? The last sample was in a previous block.

For these reasons, one often creates their own buffer. Lets start with Code example 19.5, a simple audio worklet that always delays the signal by 12 samples.

registerProcessor('fixedDelay', class extends AudioWorkletProcessor {

constructor() {

super()

this.Buffer= new Array(20).fill(0)

}

process(inputs, outputs) {

let delaySamples=12

for (let i=0;i<outputs[0][0].length;++i) {

outputs[0][0][i]=inputs[0][0][i]

for (let j=1;j<delaySamples;j++) this.Buffer[j-1]=this.Buffer[j]

this.Buffer[delaySamples-1]=inputs[0][0][i]//write at 20th sample

}

return true

}

})

Code example 19.5. fixedDelay AudioWorklet, illustrating rewriting a buffer each sampling period.

First note that we created the buffer in the constructor. Its just a simple array. It needs to have a length at least as long as the maximum delay, so we set its length to 20; the last 8 elements in the array are never used. We initialised the array to all zero values, so that we are saying there was no previous input before this worklet started.

We write input values into the buffer and read output values from the buffer. Figure 19.1 shows this buffer for a few iterations of the loop. For each iteration, the output is read from the first cell in the array. Then, values in each cell in the array are copied from the value in the next cell in the array, overwriting the value previously stored in that cell. Finally, the current input is written to the last cell in the buffer.

Figure 19.1. Rewriting a buffer to store previous inputs.

Of course, this is not the way we want to do it. Here, we are rewriting the buffer with every new sample. Its very computational, an Order N2 operation, to process one buffer of N samples. But each sample period  we really only want to remove the oldest sample and add one new sample. So this is a good place to introduce circular buffers with read and write pointers.

Here, we only keep track of a write pointer, whose value is the array index where we write data, and a read pointer, whose value is the array index where we read data. Each sampling period, the write pointer and read pointer are incremented. It’s a circular buffer because when a pointer reaches the end of the array, it wraps around to the beginning. So here’s the same code as before, but now implemented with read and write pointers.

registerProcessor('fixedDelay2', class extends AudioWorkletProcessor {

constructor() {

super()

this.Buffer= new Array(48000).fill(0)

this.ReadPtr=0,this.WritePtr=0

}

process(inputs, outputs) {

let delaySamples= 12

bufferSize=this.Buffer.length

for (let i=0;i<outputs[0][0].length;++i) {

while (this.WritePtr>bufferSize) this.WritePtr-=bufferSize

this.ReadPtr=this.WritePtr-delaySamples

while (this.ReadPtr<0) this.ReadPtr+=bufferSize

this.Buffer[this.WritePtr]=inputs[0][0][i]

outputs[0][0][i]=this.Buffer[this.ReadPtr]

this.WritePtr++,this.ReadPtr++

}

return true

}

})

Code example 19.6. fixedDelay2 AudioWorklet, illustrating a fixed delay using a circular buffer.

## Example – the Karplus Strong algorithm revisited

In Chapter 10, we introduced the Karplus Strong algorithm, for simulating a plucked string using feedback delay. But we have a problem. The Web Audio API automatically adds one block delay when a delay node appears in a feedback loop. So x seconds of delay is actually x + 128/fs, where fs is the sample rate. Hence for a sample rate of 48,000 samples per second, the highest frequency of a note produced using this algorithm is 48,000 / 128, or 375 Hz. This holds even when the delay is implemented with a simple audio worklet, since one frame of 128 samples is processed before it can be passed back to the input. So, the highest fundamental frequency that can be produced is fs/128 (~344.5 for 44.1kHz).

The solution is to implement the feedback loop inside an audio worklet, as shown in Code example 19.7. Note that this is very simplified compared to the full Karplus-Strong algorithm. For instance, there is no filter in the feedback loop, just a gain reduction. If we want filtering, that would need to also be part of the audio worklet.

<p>Decay</p>

<input type='range' min=0.8 max='0.999' value='0.9' step='any' id='Decay'>

<span id='DecayLabel'></span>

<p>Delay (ms)</p>

<input type='range' min=0 max=20 value=10 step='any' id='Delay'>

<span id='DelayLabel'></span>

<p>Width (ms)</p>

<input type='range' min=0 max='20' value=10 step='any' id='Width'>

<span id='WidthLabel'></span>

<p>

<input type='button' value='play' id='Play'>

<script>

DecayLabel.innerHTML = Decay.value

DelayLabel.innerHTML = Delay.value

</script>

<script src='KarplusStrongV2.js'></script>

var context = new AudioContext

context.audioWorklet.addModule('KSWorklets.js').then(() => {

let Noise = new AudioWorkletNode(context,'noise-generator'),

NoiseGain = new GainNode(context,{gain:0}).

output = new GainNode(context),

feedbackDelay= new AudioWorkletNode(context,'feedbackDelay-processor',

{parameterData:{delayTime:5,gain:0.9}

})

Noise.connect(NoiseGain)

NoiseGain.connect(output)

NoiseGain.connect(feedbackDelay)

feedbackDelay.connect(output)

output.connect(context.destination)

Decay.oninput = function() {

feedbackDelay.parameters.get('gain').value=this.value

DecayLabel.innerHTML = this.value

}

Delay.oninput = function() {

feedbackDelay.parameters.get('delayTime').value=this.value

DelayLabel.innerHTML = this.value

}

Width.oninput = function() { WidthLabel.innerHTML = this.value}

Play.onclick = function() {

context.resume()

var newDelay= Number(Delay.value)+1000*128/context.sampleRate

feedbackDelay.parameters.get('delayTime').value= newDelay

let now = context.currentTime

NoiseGain.gain.setValueAtTime(0.5, now)

NoiseGain.gain.linearRampToValueAtTime(0, now + Width.value/1000)

}

})

registerProcessor('feedbackDelay-processor', class extends AudioWorkletProcessor {

static get parameterDescriptors() { return [

{name:'gain',defaultValue:0.9,minValue:-1,maxValue:1},

{name:'delayTime',defaultValue:10,minValue:0,maxValue:1000}]

}

constructor() {

super()

this.Buffer= new Array(48000).fill(0)

this.ReadPtr=0,this.WritePtr=0

}

process(inputs, outputs, parameters) {

let delaySamples=Math.round(sampleRate*parameters.delayTime[0]/1000),

bufferSize=this.Buffer.length

for (let i=0;i<outputs[0][0].length;++i) {

outputs[0][0][i]= parameters.gain[0]*this.Buffer[this.ReadPtr]+inputs[0][0][i]

this.Buffer[this.WritePtr]=outputs[0][0][i]

this.WritePtr++

if (this.WritePtr>=bufferSize) this.WritePtr=this.WritePtr-bufferSize

this.ReadPtr=this.WritePtr-delaySamples

if (this.ReadPtr<0) this.ReadPtr=this.ReadPtr+bufferSize

}

return true

}

})

Code example 19.7. The Karplus Strong algorithm using an audioworklet for the feedback delay.

Code example 19.7 shows an audio worklet that implements a feedback delay node. It has two parameters, the feedback gain, which must be strictly less than 1 to avoid stability issues, and delayTime, given in milliseconds. It stores one circular buffer (a delay line) for the output. For each new sample x[n], we output y[n]=gy[n-d]+x[n], where g is the gain and d is the delay in samples, and n is the sample number.